Used	Hardinge Bridgeport XR600 5 Axis Machining Centre

Serial Number:	UX60B020

Year of Manufacture:	2008

| SPECIFICATION |  |
| --- | --- |
| Axis Travel |  |
| Table (X axis) | 885mm (-580, +305) |
| Saddle (Y axis) | 800mm (-375, +425) |
| Head (Z axis) | 500mm |
| Spindle nose to table at 0 degrees | 130~630mm |
| Spindle nose to centre of table at 90 degrees | 160~660mm |
| A Axis |  |
| Minimum resolution | 0.0001° |
| Swing range of A axis | +30/-120° |
| Speed | 25rpm |
| Clamping force | 1666Nm |
| C Axis |  |
| Minimum resolution | 0.0001° |
| Turning Range of C axis | 360° |
| Speed | 33.3rpm |
| Clamping force | 2156Nm |
| Positioning |  |
| Auto mode (X and Y axes) | 36m/min |
| Auto mode (Z axis) | 36m/min |
| Manual mode (X, Y & Z axes) | 2000mm/min |
| Feedrate range ( X and Y axes) | 0-15m/min |
| Feedrate range (z axis) | 0-15m/min |
| Acceleration X/Y/Z | 5/5.5/5.5 m/s² |
| Minimum increment | 0.001mm |
| Ball screw diameter and pitch (X, Y) | 45 x 12mm |
| Ball screw diameter and pitch (Z) | 45 x 12mm |
| Feed force N (X/Y/Z) | 13,700/13700/13,700N |
| Spindle |  |
| Spindle speed range belted | 0-12,000rpm |
| Spindle motor rating (S6-40%) | 25Kw |
| Spindle torque 12,000rpm (S6-40%) | 159Nm |
| Retention force | 12kN |
| Spindle taper | No. 40 |
| Tool holder | CAT40 |
| Spindle taper | Face & Taper #40 |
| Worktable |  |
| Working surface | 600 x 500mm |
| Max work piece diameter & height | 600 x 450mm |
| Table load | 300kg |
| Number of T-slots/Face plate | 5 Tee slots |
| T-slot size | 14mm |

Used	Hardinge Bridgeport XR600 5 Axis Machining Centre

Serial Number:	UX60B020

Year of Manufacture:	2008

| Automatic Tool Changer |  |
| --- | --- |
| Type of tool shank | CT40 taper |
| Magazine capacity | 48 tool |
| Tool select by shortest path and random select | Bi-Directional |
| Maximum tool diameter (adjacent pockets) | 75mm |
| Maximum tool diameter (without adjacent pockets) | 125mm |
| Maximum tool length | 280mm |
| Maximum tool weight | 7kg |
| Tool change time (chip to chip) ISO 10791-9 | 4.9 seconds |
| Coolant |  |
| Coolant tank capacity | 400L |
| Machine Size |  |
| Machine height | 3,030mm |
| Floor space | 3,950 x 3,670mm |
| Mass of machine | 12,100kg |
| Service Requirements |  |
| Electrical supply (input) | Balanced 3-phase |
| Structure | 50Hz |
| Cycles | 35KVA |
| Power | 380/440 volt |
| Voltage | 5.5bar |
| Compressed air (pressure flow) | 110L/min @ 2 bar |
| Nozzle coolant | 12L/min @ 20 bar |
| Wash down | 110L/min @ 2 bar |

MACHINE EQUIPPED WITH:-

Heidenhain iTNC 530 CNC Control

Linear scales

Renishaw OPM40-2 probe

Rigid tapping

Through spindle coolant

Swarf management system

Lighting

Enclosed guarding

Manuals